# Plenary Answers: Using MMC in different scenarios

Below are different scenarios. Consider the benefits and drawbacks of using each type of construction method in each scenario.

A one-off house design within a special architectural interest conservation area.

|  | Benefits | Drawbacks |
| --- | --- | --- |
| Traditional | - Would be in keeping with the local architecture and would have to use specified materials | - Longer construction time - More waste generated in comparison to other MMC - Expensive due to the associated labour costs |
| Volumetric | - More energy efficient - Can be built off-site as a single unit or separate components, and delivered and assembled on-site - Can be creative with design if the site is difficult to access or on a sloping site | - May have issues moving the prefabricated unit to site due to traffic and road limitations - Requires heavy lifting to lift units into place |
| Panelised | - More energy efficient - Lower weight - Relatively easy to transport to site - Relatively easy to make changes to the panel design | - Will need to be clad in more traditional materials, in keeping with the materials in the local environment |

A collection of 22 identical houses for a social housing project.

|  | Benefits | Drawbacks |
| --- | --- | --- |
| Traditional | - Could be more in keeping with the local architecture - Smaller on-site workforce | - It takes more time for construction due to the intensive handling of materials - Potential delays due to weather and ground conditions |
| Volumetric | - Quick to construct - More energy efficient - Improved quality control with all units - Greater quality assurance (QA) and quality control (QC) - Fewer delays to adverse on-site weather conditions | - Lack of customisation - Expensive transportation costs - Need for heavy lifting gear on-site and potential road closures |
| Panelised | - Panels made off-site and delivered when required - Identical panels made off-site will reduce overall manufacturing costs - Less on-site waste | - Time the timber is left exposed if the panels are not clad quick enough once assembled on-site could result in moisture increase, and mould developing - Panelised components are often bulky and large, and therefore difficult and expensive to transport - Trucking and transport limitations can make it more expensive to work with panels over 8.5 feet in width, and length limitations must also be considered - Panels are pre-built, and therefore it is difficult to make adjustments on-site |

A high-rise office block with limited site access.

|  | Benefits | Drawbacks |
| --- | --- | --- |
| Traditional | - Facing materials can be used to match local buildings | - It takes more time for construction - High-rise scaffolding required adds to cost and increases risks |
| Volumetric | - Quickly constructed due to off-site manufacture of units that are delivered just-in-time (JIT manufacturing) - More energy efficient - Identical units that can be stacked | - If access is limited, e.g. in terms of the width of roads, there may be an issue supplying volumetric units - High-rise lifting gear required |
| Panelised | - Potentially fewer access issues than volumetric as panels will be smaller than whole units - Small design and manufacture changes can be made with panels | - Need more assembling on-site |

A single-storey building for an outdoor pursuits centre within a dense forest that can only be closed to the public for two weeks.

|  | Benefits | Drawbacks |
| --- | --- | --- |
| Traditional | - Possibly in keeping with the look of other buildings in the area - Can use a range of materials so the building is more in keeping | - Will take longer than two weeks to construct - A lot of vehicles coming and going may damage the area |
| Volumetric | - May be able to construct within timeframe – off-site and delivered just-in-time (JIT manufacturing) - Less waste so potentially more within the ethos of the client | - May be issues with access if it is a dense woodland - May not be in keeping with area |
| Panelised | - Easier to transport through dense woodland - Panels can be manufactured ahead of the two-week shutdown and delivered to site on day one of the project - Smaller panels can be used, delivered and assembled on-site | - May not be in keeping with area - Woodland roads may not be suitable for the type of vehicle required to lift large panels - Lifting gear may not be able to get through the woodland - Height required to lift panels may be restricted by surrounding trees and vegetation |